DISCHARGE SDR 35

The SDR 35 are safe discharge rods, designed to conduct the safe damp discharge of residual charge on a cable or piece of electrical equipment with voltages up to 35kV respectively by limiting the discharge current. The SDR 35 features high grade glass fibre tubing and incorporates encapsulated resistor chains and are supplied with flexible cable with a heavy duty brass clamp for easy earth connection.

TECHNICAL SPECIFICATION

- Discharge voltage: 35kV AC/DC Maximum

- Peak Discharge current: 3.5A for 100ms

- Bleeder Resistance: 10K ohm

- Discharge Capacitance: 6uF

- Leakage Current at 35kV AC: <50uA

- Discharge Energy: 3675 J

- Cooling; Neutral

- Total Size: 32mmX950mm

- Weight: 2.2kg(net)

- Temperature:-5 Celsius to 70 Celsius.

- Safety: IEC61230, EMC Not Applicable

- Safe HV discharge rod for safe damp discharge of cables and / or equipment under test

- Supplied with flexible cable with heavy duty brass clamp for easy earth connection

- Robust single pole device

- No switches

- Suitable for voltages to 11kV/35kV

- Supplied with cushion soft carrying case

STANDARD ACESSORIES

- Earthling wire 10 sq.mm x 5mtr

- Yellow green wire

- Carry case

- Manual